# Chronos Review

## General

Assumption – logic design is correct.

## Power supplies

### Analog VCCA 1.0 V Power For DPHY And Serdes

U14 TLV73310PDQNT

Footprint should be fixed accordingly to datasheet

### DPHY, Serdes, ADC And VCCAUX Analog 1.8 V Power

U13 BD00IC0WHFV-GTR

Can be used up to 50C when 0.9W is consumed

Can be used up to 75C because of we use 8 layer pcb

### 5.0V, 3.3V

U15 LT3508EUF - OK

### Core Power 1.0V, Core Power 1.8V

U16 LT3508EUF - OK

## Power CSI and Banks

- Why not using N-CH FET instead of NPN? It consumes less current and more common

- is only used to drive LEDs; it is not necessary

- U11, U12 – Fix footprint according to datasheet

## Camera Interface

- I think FB2, FB3, FB9, FB10 are not required. Why not connecting directly to GND?

A ferrite bead is used because it is an analog ground. If no problems are observed during testing, a 0R resistor can be installed.

## Config and ADC

- Why not using N-CH FET instead of NPN? It consumes less current and more common

- is only used to drive LEDs; it is not necessary

## LAYOUT

There are many violations. Most of footprints should be corrected.

Please use the following rules to fix those.

- Solder mask opening for pads shall be 2mil max, 0mil in case of high density.

- Solder mask slivers shall be larger than 3mil

- Clearances:

- Clearances are configured according to JLCPCB capabilities

- Clearance from board edge

- JLCPCB min board edge clearence is 0.2mm

  - Pads shall be greater than 40mil from edge

  - Copper shall be greater than 20mil from edge

- Polygon connection

  - Via – direct connection

  - Pad – Relief connection

Only the GNDs are connected in relief.

- Minimum angular ring = 5mil

- Made according to JLCPCB specifications

- Acute angle = >60

- this rule is not working properly

- Silk to solder mask clearance = 5mil

- ok