# PROJECT PROPOSAL

Multi-Camera MIPI Synchronization System

for NVIDIA Jetson Orin NX Platform

| Prepared For: | Dov Katz |
| --- | --- |
| Prepared By: | Umit Kayacik |
| Date: | December 11, 2025 |
| Version: | 2.0 |

# 1. Executive Summary

This proposal outlines the development of a custom PCB system designed to synchronize four MIPI cameras and aggregate their image streams into a single 2-lane MIPI CSI-2 output for delivery to an NVIDIA Jetson Orin NX platform. The system utilizes a Lattice CrossLink-NX FPGA for camera stream aggregation via virtual channels, enabling all four synchronized cameras to share a single CSI port. Hardware-triggered synchronization ensures sub-microsecond accuracy, with zero-copy transfer of synchronized frames directly into GPU memory for computer vision processing.

# 2. Project Overview

## 2.1 Objectives

- Design and fabricate a custom PCB to interface 4x OV9281 MIPI cameras

- Implement hardware-level synchronization via trigger signals

- Aggregate 4 camera streams into single 2-lane MIPI output using Lattice CrossLink-NX FPGA

- Develop kernel-level driver for zero-copy GPU memory transfer

- Integrate IMU with synchronized trigger signal

- Design for commercial-grade components to optimize cost for mass production

## 2.2 System Constraints

- Available Jetson Interface: Single 2-lane MIPI CSI-2 port (other ports occupied by existing sensors)

- Solution: Lattice CrossLink-NX FPGA for 4-to-1 MIPI virtual channel aggregation

- Component Grade: Commercial temperature range (-10°C to +55°C) - no extreme environment requirements

## 2.3 Camera Specifications

| Parameter | Specification |
| --- | --- |
| Camera Module | KLT-Z6MF-OV9281 V3.0 |
| Sensor | OmniVision OV9281 (Global Shutter) |
| Resolution | 1MP (1280 x 800) |
| Interface | MIPI CSI-2 (2-lane per camera) |
| Quantity | 4 units |
| Cable Length | 10-15cm (MIPI direct connection) |

# 3. Technical Architecture

## 3.1 System Block Diagram

The proposed system architecture:

- 4x OV9281 Camera Modules → MIPI CSI-2 (2-lane each) →

- Lattice CrossLink-NX FPGA (4:1 aggregation with VC tagging) →

- Single 2-Lane MIPI CSI-2 Output → Jetson Orin NX

- Hardware Trigger Generator → Simultaneous sync to all cameras + IMU

## 3.2 Hardware Design

PCB Design Tool: Altium Designer

### 3.2.1 Lattice CrossLink-NX FPGA Solution

The CrossLink-NX is selected as the mandatory aggregation solution due to single available CSI port constraint:

- Device: Lattice CrossLink-NX (LIFCL-40 or LIFCL-17) - latest generation with improved performance

- MIPI D-PHY: Hardened D-PHY IP supporting up to 2.5 Gbps/lane

- Input Configuration: 4x CSI-2 RX interfaces (2-lane each) for camera inputs

- Output Configuration: 1x CSI-2 TX interface (2-lane) to Jetson Orin NX

- Virtual Channel Mapping: Camera 1→VC0, Camera 2→VC1, Camera 3→VC2, Camera 4→VC3

- Sync Integration: FPGA generates precision trigger signal distributed to all cameras simultaneously

- Future Expansion: FPGA can output additional sync signals for external sensors if needed in V2

### 3.2.2 MIPI CSI-2 Interface Design

- Differential Pair Routing: 100Ω differential impedance with tight length matching (±0.1mm within pairs)

- Lane Configuration: Each camera uses 2 data lanes + 1 clock lane (D0±, D1±, CLK±)

- Data Rate: Up to 800 Mbps/lane per camera input

- ESD Protection: Low-capacitance TVS diodes (<0.5pF) on all MIPI lines

- Connectors: Commercial-grade 15-pin FPC connectors

### 3.2.3 Synchronization Trigger Circuit

- Trigger Source: FPGA-generated precision timing signal (CrossLink-NX internal PLL)

- Distribution: Low-skew buffer for simultaneous trigger to all 4 cameras and IMU

- Camera Interface: OV9281 FSIN (Frame Sync Input) for external trigger mode

- Skew Specification: <100ns between all trigger outputs

- Frame Rate: Configurable up to 120fps

### 3.2.4 IMU Integration

- Recommended IMU: BMI088 or ICM-42688-P (6-axis, commercial grade)

- Sync Method: Same FPGA trigger pulse routed to IMU interrupt pin

- Interface: SPI (up to 10 MHz)

### 3.2.5 Power Management

- Camera Power: 1.8V/2.8V/1.2V rails (~150mW per camera)

- FPGA Power: 1.0V core, 1.8V I/O, 2.5V PLL

- Total Power Budget: ~2.5W for entire board

## 3.3 Software/Firmware Development

### 3.3.1 FPGA Firmware

- Development Tool: Lattice Radiant Software

- CSI-2 RX Controllers: 4x receivers with packet parsing and VC extraction

- Frame Buffer: Internal SRAM for line buffering during aggregation

- CSI-2 TX Controller: Packet reconstruction with VC tagging

- Trigger Generator: Precision timing with configurable frame rate

### 3.3.2 Kernel Driver Architecture

- V4L2 Subdevice Driver: OV9281 sensor control (exposure, gain, trigger mode)

- NVCSI Integration: Interface with NVIDIA camera subsystem

- VC Demultiplexing: Separate VC0-VC3 streams into /dev/video0-3

- Device Tree: Custom DTS overlay for Orin NX

### 3.3.3 Zero-Copy GPU Memory Transfer

- NvBuffer API: DMA-capable memory allocation

- CUDA-EGL Interop: Direct mapping to CUDA memory without CPU copy

- Buffer Queue: Triple-buffering to prevent frame drops

# 4. Scope of Work

- Requirements Analysis: System requirements and interface specifications

- Schematic Design: Complete circuit design with CrossLink-NX FPGA

- PCB Layout: High-speed design with proper MIPI impedance control

- Fabrication: PCB manufacturing and component procurement

- Assembly: Board assembly and hardware testing

- FPGA Firmware: CrossLink-NX firmware development

- Kernel Driver: Linux driver and zero-copy implementation

- Integration & Testing: Full system validation

- Documentation: Technical documentation package

# 5. Deliverables

- Complete schematic design files (Altium Designer)

- PCB layout files with Gerber outputs

- Bill of Materials (BOM)

- 3x Working prototype boards (assembled and tested)

- FPGA firmware source code (Lattice Radiant project)

- Kernel driver source code

- Sample application demonstrating zero-copy GPU memory access

- Technical documentation package

# 6. Project Timeline Options

## 6.1 Standard Timeline (10-12 Weeks)

| Phase | Task | Duration |
| --- | --- | --- |
| 1 | Requirements & Architecture | Week 1 |
| 2 | Schematic Design | Weeks 2-3 |
| 3 | PCB Layout & Review | Weeks 3-4 |
| 4 | PCB Fabrication & Procurement | Weeks 5-6 |
| 5 | Assembly & Hardware Testing | Week 7 |
| 6 | FPGA Firmware + Kernel Driver | Weeks 7-9 |
| 7 | Integration & Testing | Weeks 10-11 |
| 8 | Documentation & Delivery | Week 12 |

## 6.2 Expedited Timeline (4-5 Weeks)

Accelerated delivery with dedicated priority focus:

| Phase | Task | Duration |
| --- | --- | --- |
| 1 | Requirements + Schematic (parallel) | Days 1-5 |
| 2 | PCB Layout (expedited review) | Days 6-10 |
| 3 | PCB Fab (express) + FPGA dev (parallel) | Days 11-18 |
| 4 | Assembly + Driver dev (parallel) | Days 19-25 |
| 5 | Integration, Testing & Delivery | Days 26-35 |

Expedited option includes: Express PCB fabrication, parallel development tracks, dedicated priority scheduling, daily progress updates.

# 7. Budget Estimate

## 7.1 Development Cost (NRE)

| Item | Standard | Expedited |
| --- | --- | --- |
| Hardware Design (Schematic + PCB) | $6,000 | $10,800 |
| PCB Fabrication (3 boards) | $800 | $1,800 |
| Components & Assembly (3 units) | $1,500 | $2,200 |
| FPGA Firmware Development | $3,000 | $4,800 |
| Kernel Driver & Software | $2,500 | $3,800 |
| Integration, Testing & Docs | $1,200 | $1,800 |
| TOTAL NRE | $15,000 | $25,200 |

## 7.2 Mass Production Cost Estimate (Per Board, Excluding Cameras)

| Production Volume | Estimated Unit Cost |
| --- | --- |
| Prototype (5 units) | ~$400/unit |
| Small batch (10-50 units) | ~$150-200/unit |
| Medium volume (100-500 units) | ~$100-120/unit |
| High volume (1000+ units) | $70-90/unit |

Note: Mass production estimates based on commercial-grade components, China-based manufacturing partners. Final pricing depends on component availability and exact BOM. Camera modules (KLT-Z6MF-OV9281) and Jetson Orin NX not included.

# 8. Assumptions & Clarifications

- Single 2-lane MIPI CSI-2 port available on Jetson (CrossLink-NX aggregation required)

- Camera-to-board distance: 10-15cm (MIPI direct, no serializer needed)

- Commercial-grade components (-10°C to +55°C operating range)

- Client provides camera modules and Jetson Orin NX for development

- IMU selection finalized during requirements phase

- No display output - GPU memory processing for computer vision

- Future V2 may include additional sync outputs for external sensors

# 9. Payment Terms

- 30% upon project kickoff

- 30% upon schematic and PCB design approval

- 40% upon final delivery and acceptance

# 10. Contact Information

Umit Kayacik

Senior Electronics Engineer

I look forward to working with you on this project. Please don't hesitate to reach out if you have any questions or require additional information.