# SYSTEM REQUIREMENTS DOCUMENT

Project Chronos

Multi-Camera MIPI Synchronization System

for NVIDIA Jetson Orin NX Platform

Prepared By: Umit Kayacik

Document Version: 1.0

Date: December 29, 2025

# 1. Executive Summary

This document defines the complete system requirements for the Chronos project - a custom hardware and software solution designed to synchronize four OV9281 MIPI cameras and aggregate their image streams into a single 2-lane MIPI CSI-2 output for delivery to an NVIDIA Jetson Orin NX platform. The system utilizes a Lattice CrossLink-NX FPGA for camera stream aggregation via virtual channels, enabling all four synchronized cameras to share a single CSI port. Hardware-triggered synchronization ensures sub-microsecond accuracy, with zero-copy transfer of synchronized frames directly into GPU memory for computer vision processing.

# 2. Project Objectives

- Design and fabricate a custom PCB to interface 4x OV9281 MIPI cameras

- Implement hardware-level synchronization via trigger signals

- Aggregate 4 camera streams into single 2-lane MIPI output using Lattice CrossLink-NX FPGA

- Develop kernel-level driver for zero-copy GPU memory transfer

- Integrate IMU (ICM-42688-P) with synchronized trigger signal

- Publish synchronized images using NVIDIA NITROS node

- Publish IMU data using ROS2 node with FPGA timestamps

- Design for commercial-grade components to optimize cost for mass production

# 3. System Constraints

## 3.1 Interface Constraints

- Available Jetson Interface: Single 2-lane MIPI CSI-2 port (other ports occupied by existing sensors)

- Solution: Lattice CrossLink-NX FPGA for 4-to-1 MIPI virtual channel aggregation (MANDATORY)

- Total Camera Requirement: 7 cameras total (4 synchronized OV9281 + 3 TBD higher-res cameras)

- Available MIPI Ports: 4x 2-lane ports on Jetson Orin NX (1 for aggregated cameras, 3 for other sensors)

## 3.2 Environmental Constraints

- Component Grade: Commercial temperature range (-10°C to +55°C)

- No Extreme Conditions: No operation below -20°C required (Jetson limitation)

- Commercial Application: Cost-optimized components acceptable

## 3.3 Physical Constraints

- Camera-to-Board Distance: 10-15cm (within MIPI direct connection range of 30cm)

- No Serializer Required: FPD-Link or GMSL not needed for this application

# 4. Hardware Requirements

## 4.1 Camera Module Specifications

| Parameter | Specification |
| --- | --- |
| Camera Model | SF-C1014OV-B&W-H80 |
| Sensor | OmniVision OV9281 (Global Shutter) |
| Resolution | 1MP (1280 x 800) |
| Pixel Size | 3.0 μm × 3.0 μm |
| Sensor Size | 1/4 inch |
| Interface | MIPI CSI-2 (2-lane per camera) |
| Connector | AXT624124 (24-pin FPC) |
| Frame Rate (Full Res) | Up to 120 fps |
| Frame Sync | FSIN (Frame Sync Input) - Pin 16 |
| Focal Length | 2.2mm |
| F-Number | F2.0 |
| FOV (Horizontal) | 80° |
| TV Distortion | < -1% |
| Focusing Range | 60cm to infinity |
| Quantity | 4 units |
| Cable Length | 10-15cm (MIPI direct connection) |

## 4.2 FPGA Requirements (Lattice CrossLink-NX)

| Parameter | Specification |
| --- | --- |
| Device | Lattice CrossLink-NX (LIFCL-40 or LIFCL-17) |
| Technology | 28nm FD-SOI (Nexus platform) |
| MIPI D-PHY | Hardened D-PHY IP, 2.5 Gbps/lane, max 10 Gbps aggregate |
| Input Configuration | 4x CSI-2 RX interfaces (2-lane each) |
| Output Configuration | 1x CSI-2 TX interface (2-lane) to Jetson |
| Virtual Channel Mapping | Camera 1→VC0, Camera 2→VC1, Camera 3→VC2, Camera 4→VC3 |
| Reference Design | 4-to-1 Image Aggregation with CrossLink-NX |
| Development Tool | Lattice Radiant Software |

## 4.3 IMU Requirements

| Parameter | Specification |
| --- | --- |
| IMU Model | ICM-42688-P |
| Type | 6-axis MEMS (3-axis gyro + 3-axis accelerometer) |
| Package | 2.5mm x 3mm x 0.91mm 14-pin LGA |
| Interface | SPI (up to 24 MHz) / I2C (up to 1 MHz) |
| Gyroscope Range | ±15.625 to ±2000 dps (programmable) |
| Accelerometer Range | ±2g, ±4g, ±8g, ±16g (programmable) |
| FIFO | 2KB buffer |
| Sync Method | FPGA trigger pulse routed to interrupt pin |
| Features | External clock input (31-50kHz), programmable interrupts |

## 4.4 Carrier Board (Helios by Aerium)

| Parameter | Specification |
| --- | --- |
| Board Model | Aerium Helios |
| Dimensions | 59mm x 89.5mm |
| Weight | 40g (without Jetson module) |
| Input Voltage | 12-48V DC |
| Compatible Modules | Jetson Orin NX, Orin Nano, Xavier NX, TX2 NX, Nano |
| Camera Interface | Quad MIPI camera interface |
| Features | SuperMode support, Hardware TPM, expanded USB 3.0 |
| PCIe Interface | Available (currently used for NVMe SSD) |

## 4.5 Synchronization Requirements

### 4.5.1 Trigger Circuit Specifications

- Trigger Source: FPGA-generated precision timing signal (CrossLink-NX internal PLL)

- Distribution: Low-skew buffer for simultaneous trigger to all 4 cameras and IMU

- Camera Interface: OV9281 FSIN (Frame Sync Input) for external trigger mode

- Trigger Pulse Width: Minimum 2μs (as per OV9281 specification)

- Trigger Voltage Level: 3.3V

- Skew Specification: <100ns between all trigger outputs

- Frame Rate: Configurable up to 120fps

- 5th Camera Sync: Additional sync signal output for external camera (without aggregation)

### 4.5.2 Timestamp Requirements

- Timestamp Source: FPGA-based hardware timer

- Resolution: Microsecond precision

- Synchronization: Identical timestamp applied to all triggered devices (cameras + IMU)

## 4.6 PCB Design Requirements

- Design Tool: Altium Designer

- MIPI Routing: 100Ω differential impedance with tight length matching (±0.1mm within pairs)

- Lane Configuration: Each camera uses 2 data lanes + 1 clock lane (D0±, D1±, CLK±)

- Data Rate: Up to 800 Mbps/lane per camera input

- ESD Protection: Low-capacitance TVS diodes (<0.5pF) on all MIPI lines

- Connectors: Commercial-grade 15-pin FPC connectors

## 4.7 Power Management Requirements

- Camera Power: 1.8V/2.8V/1.2V rails (~150mW per camera)

- FPGA Power: 1.0V core, 1.8V I/O, 2.5V PLL

- Total Power Budget: ~2.5W for entire board

# 5. Software Requirements

## 5.1 FPGA Firmware Requirements

- CSI-2 RX Controllers: 4x receivers with packet parsing and VC extraction

- Frame Buffer: Internal SRAM for line buffering during aggregation

- CSI-2 TX Controller: Packet reconstruction with VC tagging

- Trigger Generator: Precision timing with configurable frame rate

- Timestamp Counter: Hardware timer for synchronized timestamps

- I2C Master: Camera sensor configuration interface

## 5.2 Linux Driver Requirements

### 5.2.1 Kernel Driver Architecture

- V4L2 Subdevice Driver: OV9281 sensor control (exposure, gain, trigger mode)

- NVCSI Integration: Interface with NVIDIA camera subsystem

- VC Demultiplexing: Separate VC0-VC3 streams into /dev/video0-3

- Device Tree: Custom DTS overlay for Orin NX

### 5.2.2 Zero-Copy GPU Memory Transfer

- NvBuffer API: DMA-capable memory allocation

- CUDA-EGL Interop: Direct mapping to CUDA memory without CPU copy

- Buffer Queue: Triple-buffering to prevent frame drops

## 5.3 ROS2 Integration Requirements

### 5.3.1 Camera Node (NITROS)

- Framework: NVIDIA Isaac ROS NITROS

- Zero-Copy Support: Type adaptation and type negotiation for GPU memory sharing

- Message Type: isaac_ros_nitros_image_type

- Topics: 4 synchronized image streams with FPGA timestamps

- Performance: Full resolution (1280x800) at 60fps minimum

### 5.3.2 IMU Node

- Framework: Standard ROS2 node

- Message Type: sensor_msgs/Imu

- Interface: SPI communication with ICM-42688-P

- Timestamp: FPGA timestamps included in message header

- FSYNC Flag: Trigger event marker in IMU data stream

# 6. Performance Requirements

| Requirement | Specification |
| --- | --- |
| Image Resolution | 1280 x 800 (1MP) |
| Target Frame Rate | 60 fps (synchronized) |
| Maximum Frame Rate | 120 fps (OV9281 capability) |
| Synchronization Accuracy | < 100ns skew between cameras |
| Timestamp Resolution | Microsecond precision |
| Memory Transfer | Zero-copy to GPU memory |
| Latency (Glass-to-GPU) | Minimize - no display output |
| IMU Sample Rate | Synchronized with camera trigger |

# 7. Deliverables

## 7.1 Hardware Deliverables

- Complete schematic design files (Altium Designer)

- PCB layout files with Gerber outputs

- Bill of Materials (BOM)

- 3x Working prototype boards (assembled and tested)

## 7.2 Software Deliverables

- FPGA firmware source code (Lattice Radiant project)

- Kernel driver source code

- ROS2 NITROS camera node for synchronized images

- ROS2 IMU node with FSYNC support

- Sample application demonstrating zero-copy GPU memory access

- Technical documentation package

## 7.3 Demonstration Requirements

- Demo 1: Zero-copy transfer of synced images at full resolution (1280x800) and 60fps mapped to GPU memory

- Demo 2: IMU stream readout on Jetson including FPGA timestamps

- Demo 3: FSYNC flag indication when trigger signal rises

# 8. Reference Links

- Aerium Helios Carrier Board: https://aerium.co.il/products/helios

- Helios Documentation: https://docs.aerium.co.il/Product%20Manuals/Helios/

- Lattice CrossLink-NX: https://www.latticesemi.com/Products/FPGAandCPLD/CrossLink-NX

- 4-to-1 Image Aggregation Reference Design: https://www.latticesemi.com/products/designsoftwareandip/intellectualproperty/referencedesigns/referencedesign04/4to1

- NVIDIA Isaac ROS NITROS: https://nvidia-isaac-ros.github.io/concepts/nitros/index.html

- OV9281 Sensor: https://www.ovt.com/products/ov9281/

- ICM-42688-P IMU: https://invensense.tdk.com/products/motion-tracking/6-axis/icm-42688-p/

# 9. Document History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | Dec 29, 2025 | Umit Kayacik | Initial Release |